# SolFlex 3000 Microinverter — Product Factsheet

## Overview

The SolFlex 3000 is a grid-tied solar microinverter designed for residential rooftop installations. It converts DC power generated by individual solar panels into grid-compatible AC power, offering panel-level monitoring and improved system resilience compared to traditional string inverter designs.

## Technical Specifications

| Specification | Value |
| --- | --- |
| Rated Output Power | 300 W |
| Peak Efficiency | 98.6% |
| Input Voltage Range | 16V – 60V DC |
| Output Voltage | 230V AC (configurable for 110V regions) |
| Operating Temperature Range | -25°C to 60°C |
| Weight | 1.2 kg |
| Warranty Period | 12 years standard, extendable to 20 years |
| Communication Protocol | Zigbee mesh network with cloud monitoring |
| IP Rating | IP67 (fully weatherproof) |

## Installation Notes

Each SolFlex 3000 unit should be mounted directly beneath its corresponding solar panel using the supplied mounting bracket. Installers must ensure a minimum clearance of 2 cm between the inverter housing and the roof surface to allow for adequate passive cooling airflow.

The Zigbee mesh network supports a maximum of 50 microinverters per monitoring gateway. For installations exceeding 50 panels, an additional gateway unit must be installed, with overlapping mesh coverage recommended to maintain communication reliability.

## Safety Certifications

The SolFlex 3000 has been certified to IEC 62109-1 and IEC 62109-2 safety standards for power converters used in photovoltaic systems. It also holds BIS certification (IS 16221) for sale within the Indian market and CE certification for European Union markets.

## Warranty and Support

Solara Dynamics provides a standard 12-year limited warranty covering manufacturing defects, with an optional extended warranty available up to 20 years at the time of purchase. Customers experiencing a unit failure under warranty should contact regional support with the unit's serial number and installation date for expedited replacement processing, typically completed within 5 business days.